CVS Health Network Infrastructure Analysis

IP Network Inventory, Functional Analysis & NAC Assessment

Based on: configs.zip — 28 Cisco IOS device configurations across 3 sites

# Executive Summary

This report analyzes 28 Cisco IOS network device configurations across three CVS Health corporate locations: 100 Scenic View Drive (Rhode Island corporate campus and store lab), 1CVS (Woonsocket HQ data center and internet edge), and 2100 (Rhode Island / McKesson primary data center). The analysis extracts all routed IP networks, identifies their functional roles, and evaluates evidence of Network Access Control (NAC) implementation.

All sites share a common TACACS+ authentication infrastructure using Cisco ISE (CVSHealth-ISE) with redundant nodes in Rhode Island and Arizona. OSPF is used for internal routing at the campus level, BGP at the WAN/DC edge. No active 802.1X/MAB NAC enforcement was found in these configurations — only ISE for AAA/TACACS device management is present.

# Site 1: 100 Scenic View Drive — Corporate Campus & Store Lab

## Site Overview

This site represents the CVS Health Rhode Island corporate campus. It contains a dual-MDF distribution layer (mdf-01 / mdf-02 as OSPF Area 0 cores), multiple IDF access switches, a dedicated DC lab segment (W121100), a WiFi distribution switch, and a Store Lab router used for testing connectivity to CVS retail stores (TS-68700 to TS-68748). WAN connectivity is provided by four ISR WAN-MPLS routers in separate racks (3, 5, 7, 9) each with a dedicated broadband uplink. A Verizon MPLS circuit (100Mbps) connects the DC lab to the corporate MPLS backbone.

Network Inventory

| VLAN / Interface | IP Network / Address | Description / Function | Notes |
| --- | --- | --- | --- |
| Vlan 2001 | 10.51.160.0/23 & 10.51.162.0/23 | Corporate Data / Management | Mgmt backbone; IDF/MDF uplinks and device mgmt IPs across racks 3,5,7,9 and MDF switches |
| Vlan 2001 (IDF2) | 10.51.164.0/23 | Corp Access Data VLAN | User-facing access VLAN at IDF2; DHCP via 10.228.144.15/20 and 10.18.106.117 |
| Vlan 2002 (IDF2) | 10.215.74.0/23 | Voice VLAN | VoIP VLAN at IDF2; DHCP forwarded to ISE DNS servers |
| Vlan 110 (DC Lab) | 10.51.170.0/26 | Data – DC Lab | User-facing data segment in the dist-center lab (W121100); helper to corp DNS |
| Vlan 120 (DC Lab) | 10.51.170.64/26 | Wireless – DC Lab | Wireless/WLAN segment in DC lab; connected to WLC |
| Vlan 192 (DC Lab) | 10.51.170.128/26 | Management – DC Lab | OOB/mgmt VLAN for DC lab devices |
| Vlan 311 (DC Lab) | 10.51.170.192/29 | WAN Transit – DC Lab | Transit link to MPLS router (rricrcp-mpls-distcenterlab-01) |
| Vlan 2001 (WiFi-IDF2) | 10.51.185.0/24 | Wireless Subnet | AP-facing wireless subnet at WiFi IDF2; DHCP to corp DNS + 10.18.106.117 |
| Loopback 0 (acc-idf2) | 10.51.0.164/32 | Network Management | Management loopback for OSPF/TACACS sourcing at IDF2 |
| P2P Links (MDF-01↔acc) | 10.17.51.x–10.17.52.x /30 | L3 Uplinks | Point-to-point /30s between MDF-01/02 and access/IDF switches; OSPF Area 100.0.0.6 |
| P2P (MDF↔Core/DC) | 10.0.10.26/30, 10.0.138.18/30 | Core Uplinks | Links from MDF distribution toward RI/MC data center cores; OSPF Area 0 |
| Rack WAN Transit VLANs | 100.100.30-90.0/24 (per rack) | Store WAN Transit | One /24 per rack (30,50,70,90) bridging WAN MPLS router to distribution switch; default route via 100.100.x.1 |
| MPLS WAN (Rack5 Public) | 98.185.91.194/28 | Broadband Uplink | Public-facing broadband on Rack 5 WAN router; ISP gateway .193 |
| MPLS Verizon Circuit | 10.199.30.132/30 | WAN MPLS Provider | Verizon MPLS CKT ID 019186 [C1217700] 100Mbps; used by distcenterlab router |
| BGP Advertised (dclab) | 10.51.168.0/23, 10.51.170.0/23 | BGP Summarized Prefixes | Summarized into Verizon MPLS BGP AS 64534 |
| Store Lab P2P | 172.23.254.x – 172.23.255.x /30 | Store Router Uplinks | Individual /30 uplinks to TS-68700–68748 store routers (Cisco + 128T); EIGRP/BGP AS 65502 |
| Store Lab Loopback | 172.23.131.13/32 | Store Lab EIGRP/BGP Source | BGP/EIGRP source for rricrcp-dis-mdf-storelab-01 |

# Site 2: 1CVS — Woonsocket HQ / Internet Edge

## Site Overview

The 1CVS site is the Woonsocket, RI headquarters internet and extranet edge. It houses redundant internet-facing switches (rriwsdc-hq-internet-sw01 and rriwsdc-hq-xnet-sw-01) that terminate multiple ASA firewalls, VPN concentrators (employee, B2B, client, portal, SSL), Nokia RADIUS appliances, and Cisco 4600-series VPN platforms. This is the primary public internet ingress/egress point for CVS Health. An innovation lab (Innolab) is also present with its own fabric of /30 point-to-point links and static host routes.

Network Inventory

| VLAN / Interface | IP Network / Address | Description / Function | Notes |
| --- | --- | --- | --- |
| Vlan 20 (internet-sw01) | 12.46.119.128/25 | XNET / Internet DMZ | Externally facing internet DMZ VLAN; default route to 12.46.119.234 (AT&T or provider GW) |
| Vlan 20 (xnet-sw-01) | 12.46.119.128/25 | Extranet VLAN | Redundant extranet L3; .134 on xnet-sw vs .203 on internet-sw (dual internet switches) |
| Vlan 10 | Not configured (L2 only) | Connected Device Fabric | Carries ASA firewall, VPN concentrators (ASAEMP, ASAB2B, ASAVPN), Cisco/Nokia radios |
| innolab P2P uplinks | 10.254.254.x /30 | Innovation Lab Fabric | Point-to-point links interconnecting innolab production, access, and ILO management hosts |
| innolab Uplink to DC | 10.17.11.33/30 | Innolab ↔ DC Transit | Uplink from innolab distribution switch toward main DC infrastructure |
| innolab static routes | 10.254.254.29–49/32 | Innolab Host Routes | Host /32 routes for individual innolab servers routed via 10.254.254.10 |

# Site 3: 2100 — Rhode Island / McKesson Primary Data Center

## Site Overview

The 2100 site is the primary Rhode Island data center, likely co-located with or adjacent to the McKesson/CVS pharmacy operations center. It hosts the most complex network topology: redundant WAN MPLS routers (ANT-01/02 in BGP AS 64518) with connections to AT&T, Verizon, and Lumen (CenturyLink) MPLS clouds. There are dedicated VRFs and VLANs for SIP/voice infrastructure (SBC, Omnicare pharmacy), 128 Technology (Juniper SSR) SD-WAN inside/outside segments, DMVPN, CDN source servers, and retail store internet firewall connectivity. The ptwan distribution switches (DIS11, DIS12) handle point-to-WAN routing with extensive static routing tables covering retail store, data center, and partner prefixes.

Network Inventory

| VLAN / Interface | IP Network / Address | Description / Function | Notes |
| --- | --- | --- | --- |
| Vlan 101 | 10.230.126.0/30 | RI WAN Distribution Transit | L3 transit between WAN and RI distribution layer |
| Vlan 110 | 10.230.32.0/24 | RI SBC Inside Network | Session Border Controller (SBC) internal network — voice infrastructure |
| Vlan 111 | 10.230.33.0/24 | SIP Enterprise Ops Monitor | SIP/VoIP enterprise operations monitoring |
| Vlan 112 | 10.230.34.0/24 | RI CDN Source Servers | Content Delivery Network source server segment |
| Vlan 113 | 10.230.124.240/28 | 128T Inside Subnet | 128 Technology (Juniper SSR) SD-WAN inside segment |
| Vlan 114 | 10.230.122.160/27 | 128T Outside MPLS | 128T SD-WAN outside/MPLS-facing segment |
| Vlan 115 | 10.230.122.192/27 | 128T Inside Subnet (2) | Second 128T inside subnet |
| Vlan 120 | 10.230.123.0/24 | Retail Store Internet FW Inside | Firewall inside interface for CVS retail store internet access |
| Vlan 202 | 10.230.124.40/30 | DIS7↔DIS8 SIP VRF Link | L3 interconnect between Dis7 and Dis8 in the SIP VRF |
| Vlan 230 | 10.230.124.32/29 | SIP-A FW Inside | Firewall inside for SIP-A VRF connecting to DIS7/DIS8 |
| Vlan 301 | N/A (transit) | RI PROD-DMVPN Transit | DMVPN distribution transit — connects to DMVPN hub |
| Vlan 302 | 10.230.125.128/29 | PROD-DMVPN FW Inside | Firewall inside for DMVPN production traffic |
| Vlan 303 | 10.230.125.144/28 | 128T Outside Subnet | 128T/SSR outside subnet |
| Vlan 304 | 10.230.125.160/27 | 128T Outside Internet | 128T internet-facing outside segment |
| Vlan 310 | 10.230.31.0/26 | RI SBC Omnicare Inside | Omnicare pharmacy SBC inside segment |
| ptwan-01 core links | 10.77.5.148/28, 10.134.76.0/24, 10.0.128.x/30 | RI PTWAN Fabric | Point-to-WAN distribution uplinks; connects to COR3, COR4, and peer DIS12 |
| ptwan-01 static routes | 10.77.5.x, 10.78.136.x, 10.79.241.x | PTWAN Reachability | Static routes toward PTWAN subnets via 10.77.5.145 |
| ANT-01/02 Loopbacks | 10.218.116.128–131/32 | WAN Router IDs | Individual loopbacks for iBGP peering between ANT-01 and ANT-02 |
| ANT-01 ATT MPLS | 10.218.5.133/30 | ATT MPLS Uplink | ATT MPLS circuit CKT ID IZEC.586991 to provider PE |
| ANT-02 Verizon MPLS | 10.218.5.150/30 | VZB MPLS Uplink | Verizon MPLS circuit E5010835/C0140869 |
| ANT-02 Lumen MPLS | 10.11.251.30/30 | Lumen MPLS Uplink | Lumen/CenturyLink MPLS circuit 445961972 (dual port) |
| ANT iBGP | 10.218.116.132/30 | iBGP Peering Link | Link between ANT-01 and ANT-02 for iBGP; AS 64518 |
| Anthem Context | 152.145.56.92/29 | Anthem Health Partner Link | Dedicated segment for Anthem insurance partner connectivity |
| BGP Advertised | 152.145.80.158/32, 2.2.2.2/32 | ANT Advertised Prefixes | Loopbacks and anycast advertised into BGP AS 64518 |
| Mgmt (ANT routers) | 10.17.254.240/23 | Management VRF | OOB management for ANT routers; GW 10.17.254.1 |

# NAC (Network Access Control) Assessment

## Summary Finding: NAC Not Actively Implemented on These Devices

A thorough review of all 28 configuration files for NAC-related commands and constructs found no evidence of active 802.1X or MAB (MAC Authentication Bypass) enforcement on access ports. The following table summarizes the specific findings by control area:

| NAC Control | Status | Evidence / Notes |
| --- | --- | --- |
| 802.1X (dot1x) | Absent | No 'dot1x pae authenticator', 'authentication port-control auto', or 'dot1x system-auth-control' found on any interface or globally. The 'system-cpp-police-dot1x-auth' class-map entries are Cisco IOS default CPP (Control Plane Policing) boilerplate automatically generated by IOS 17.x — they do NOT indicate dot1x is in use. |
| MAB (MAC Auth Bypass) | Absent | No 'mab' or 'authentication order mab dot1x' commands present on any switchport interface. |
| ISE / RADIUS for NAC | Partial — AAA only | Cisco ISE is configured as the TACACS+ AAA server for device management (login, enable, exec authorization, command accounting) on ALL devices. However, ISE is NOT referenced as a RADIUS server for port-based NAC. No 'radius server' stanzas or 'aaa authentication dot1x' policies found. |
| Port Security | Configured — IDF2 only | rricrcp-acc-idf2-01 has 'switchport port-security' configured on all access ports (max 2 MACs, violation restrict, aging 2min inactivity). This is a compensating control — it limits MAC flooding and rogue devices but does NOT authenticate endpoints. |
| CTS / TrustSec | Logging only | 'cts logging verbose' found only on the two PRICMDC ANT WAN routers (2100 site). No CTS policies, SGT assignments, or 'cts role-based' enforcement configured. This line enables verbose logging of CTS messages but implies no active SGT/TrustSec deployment. |
| DHCP Snooping | Not Configured | No 'ip dhcp snooping' global or per-VLAN configuration found. DHCP helper-addresses are configured on SVI interfaces for relay, but no snooping binding table protection is in place. |
| Dynamic ARP Inspection | Absent | No 'ip arp inspection' commands found. Without DHCP snooping, DAI cannot be configured (requires the snooping binding table). |
| IP Source Guard | Not Configured | No 'ip verify source' commands found on any interface. |
| Device Sensor / Profiling | Absent | No 'device-sensor' or 'access-session' global policies found — these are required for ISE profiling-based NAC. Also no 'ip device tracking' (required for IBNS 2.0). |
| WebAuth / Guest VLAN | Not Configured | No 'authentication event fail action' guest VLAN assignments or web authentication policies found on any interface. |

## Key Observations

- Cisco ISE IS deployed — but only as a TACACS+ server for device management AAA. The infrastructure (ISE nodes in RI and AZ) exists and could support full NAC if 802.1X/MAB were configured.

- Port security at IDF2 provides basic MAC-layer protection but does not authenticate users or devices against a directory or certificate store.

- The access layer at IDF2 (rricrcp-acc-idf2-01) is the most hardened access switch in the set — it has port-security, voice VLANs, spanning-tree portfast/bpduguard, and QoS policies — but lacks 802.1X.

- The store lab switches (dis-121101-rack3/5/7/9) and DC lab switch have no port-level security at all beyond the physical VLAN assignments.

- The 2100 data center switches have extensive routing complexity (SIP, DMVPN, 128T SD-WAN) but no endpoint NAC controls — appropriate for a DC environment where access is controlled at the physical/server layer.

## Recommendation

If NAC is a compliance or security requirement (HIPAA, PCI-DSS, HITRUST — all applicable to CVS Health), the existing Cisco ISE infrastructure should be extended to enforce 802.1X/MAB on access-layer switch ports. The recommended implementation path would be:

- Enable DHCP snooping and IP source guard on all access VLANs first as foundational controls.

- Configure ISE RADIUS policies and add RADIUS server stanzas to the distribution/access switches.

- Enable 'authentication port-control auto' with MAB fallback on all access ports — starting with IDF2 (already has port-security as a baseline).

- Use ISE profiling with device sensor to auto-classify endpoints before enforcing policy.

- Deploy CTS/TrustSec SGTs for lateral movement control in the data center, building on the 'cts logging verbose' already present on the ANT routers.

CVS Health Network Analysis — Confidential | Generated from configs.zip | March 2026